Droid X – Clean Reset

The follow information is intended for the Droid X in reference to applying the 2.2 upgrade over an existing 2.1 update. The same basic process can be used for the original Droid.

Entire process only takes ~10 minutes after you backup your SD card.

1. Backup your SD card.
2. Format your SD Card.
3. Power off.
4. Remove battery, Press power button to dump residual power, reinstall battery.
5. Boot into recovery mode. (Hold home button and keep it held, push power button, release power button when the "M" logo appears and continue to hold home button until you see the triangle with exclamation point. Press the search button (magnifying glass))
6. Use volume rocker to navigate to "factory reset". Select "factory reset" by pressing the camera button. Navigate to "yes" to confirm reset.
7. When reset is complete, you'll see the selection list with "reboot system" selected. Press the camera button to select the reboot.

8. Let your device boot (you should be prompted with the activate device dialer - let it activate you). Tap through until you're prompted to create, sign into or skip logging into your google account. Select "skip" at this point.
9. You should be prompted with the "Backup Assistant". Select "skip".
10. You should now be prompted with another window titled "setup accounts". Choose "done adding accounts".
11. Let your default screens load for a min...
12. Go to menu-->settings-->privacy.
13. Make sure "back up data" is checked, then make sure "restore data" is unchecked, then go back to "back up data" and uncheck it. The point here is to make sure both "back up data" and "restore data" are unchecked.
14. You can now go to menu-->settings-->accounts and log into your google account.
15. After you log into your google acount, at the bottom of the screen you'll see yet another check box that by default is checked, asking if you want to backup your data to google. I Chose no and recommend this setting for now (after this mess, I'm thinking future-proof for the next update).

Finally, done.

===================================================

Motorola Activate Phone *228 – Option 1 – Activate Phone   Option 2 – Reset Cell Towers
